DEPARTMENT OF COMPUTER SCIENCE & ENGINEERING

ASSESSMENT TOOL 2

Course Code: CSA1203 | Course Title: Computer Architecture

Debugging & Optimisation

| Student Name | P JASWANTH |
| --- | --- |
| Reg. No. | 192312437 |
| Department | ECE |

# Q1. Debugging Signed Binary Addition Using 2’s Complement

## 1. Problem Understanding

A signed binary addition program gives incorrect results when adding negative numbers represented using 2’s complement. The main causes are incorrect sign extension, incorrect overflow detection, treating the final carry as overflow, and using unsigned comparison for signed numbers.

For an n-bit 2’s complement number:

Range = -2^(n-1) to 2^(n-1) - 1

For an 8-bit system:

-128 ≤ X ≤ 127

## 2. Method Selected

Use standard binary addition with 2’s complement representation and detect overflow using the signs of the operands and result.

V = (A_MSB XOR S_MSB) AND (B_MSB XOR S_MSB)

Equivalent condition:

Overflow = (A_MSB = B_MSB) AND (S_MSB ≠ A_MSB)

Overflow occurs only when two positive numbers produce a negative result, or two negative numbers produce a positive result.

## 3. Solution Process

Consider 8-bit addition: 50 + 80

50 = 00110010
80 = 01010000

  00110010
+ 01010000
-----------
  10000010

The binary result 10000010 represents -126 in signed 8-bit 2’s complement, while the mathematical result is 130. Since 130 cannot be represented in signed 8-bit form, overflow occurs.

Overflow = 1

## 4. Debugged Logic

A common incorrect implementation is:

if carry == 1:
    overflow = 1

This is incorrect because carry-out does not necessarily indicate signed overflow. Correct logic is:

signA = A >> 7
signB = B >> 7
signS = S >> 7

overflow = (signA == signB) && (signS != signA)

## 5. Optimisation

Overflow detection can be implemented directly using the MSB bits instead of additional signed comparisons.

V = (A7 XOR S7) AND (B7 XOR S7)

This requires only XOR and AND operations and therefore provides an efficient hardware implementation.

## Result and Interpretation

The debugging shows that the problem is caused by confusing carry-out with signed overflow and by improper sign handling. Correct 2’s complement addition must preserve sign extension and use the MSB-based overflow condition.

Final Result: Correct signed addition + efficient overflow detection

# Q2. Optimisation of Carry Look-Ahead Adder

## 1. Problem Understanding

A Carry Look-Ahead Adder (CLA) is designed to be faster than a Ripple Carry Adder (RCA) because it calculates carry signals in parallel. If the expected speed improvement is not achieved, delays may exist in the generate/propagate network.

Pi = Ai XOR Bi
Gi = Ai Bi
C(i+1) = Gi + Pi Ci

## 2. Method Selected

For a 4-bit CLA:

C1 = G0 + P0C0
C2 = G1 + P1G0 + P1P0C0
C3 = G2 + P2G1 + P2P1G0 + P2P1P0C0
C4 = G3 + P3G2 + P3P2G1 + P3P2P1G0 + P3P2P1P0C0

## 3. Debugging Analysis

Generate and propagate signals are calculated sequentially.

The carry network has excessive logic depth.

Large fan-in gates are used.

Wiring delays are significant.

The implementation accidentally allows ripple propagation.

Ripple Carry Adder:

FA0 → FA1 → FA2 → FA3
Carry propagates sequentially

Carry Look-Ahead Adder:

A,B → P/G generation
        ↓
Parallel Carry Logic
 ↓    ↓    ↓    ↓
C1   C2   C3   C4

## 4. Optimisation

A hierarchical/block CLA can be used for larger adders.

Pblock = P3P2P1P0
Gblock = G3 + P3G2 + P3P2G1 + P3P2P1G0

This reduces long logic paths while maintaining approximately the same hardware complexity.

## 5. Result and Interpretation

The CLA provides faster addition because carries are calculated in parallel rather than waiting for previous carry signals. Performance can be improved by generating P and G simultaneously, using parallel carry equations, using hierarchical blocks, and avoiding unnecessarily large fan-in gates.

Conclusion: Parallel carry generation improves addition speed.

# Q3. Debugging Booth’s Multiplication Algorithm

## 1. Problem Understanding

Booth’s algorithm is used for efficient multiplication of signed binary numbers in 2’s complement representation. The algorithm examines Q0 and Q-1 and then performs an arithmetic right shift.

| Q0 Q-1 | Operation |
| --- | --- |
| 00 | No operation |
| 01 | A = A + M |
| 10 | A = A - M |
| 11 | No operation |

## 2. Example: (-5) × 3

Use 4-bit representation:

M = -5 = 1011
Q = 3 = 0011
A = 0000
Q-1 = 0

Step 1: Q0Q-1 = 10, so A = A - M. Since -M = 0101:

A = 0000 + 0101 = 0101

Perform an arithmetic right shift. Continue checking Q0Q-1 and applying the Booth decision rule for each of the four iterations.

The final 8-bit product is:

11110001

This represents:

-15

Therefore:

(-5) × 3 = -15

## 3. Debugging Errors

Error 1 – Incorrect decision rule: 01 must mean add and 10 must mean subtract.

Error 2 – Logical right shift: A signed Booth operation requires an arithmetic right shift so that the sign bit is preserved.

Error 3 – Incorrect initialization: Q-1 must initially be 0.

Error 4 – Insufficient iterations: For an n-bit multiplier, Booth’s algorithm normally requires n iterations.

## 4. Optimisation

Booth’s algorithm reduces addition/subtraction operations when the multiplier contains consecutive 1s. Modified Booth encoding (radix-4) processes two multiplier bits at a time and can approximately halve the number of iterations.

Final Result: (-5) × 3 = -15

# Q4. Debugging IEEE 754 Single-Precision Floating-Point Arithmetic

## 1. Problem Understanding

IEEE 754 single precision uses 32 bits:

| Sign | Exponent | Fraction |
| --- | --- | --- |
| 1 bit | 8 bits | 23 bits |

The exponent uses a bias of 127.

Value = (-1)^S × 1.F × 2^(E-127)

Floating-point errors can occur due to:

Incorrect exponent alignment.

Incorrect mantissa shifting.

Incorrect normalization.

Incorrect rounding.

Failure to handle carry during mantissa addition.

## 2. Method Selected

For floating-point addition:

Extract sign, exponent and fraction fields.

Compare exponents and align the smaller mantissa.

Perform mantissa addition/subtraction according to the signs.

Normalize the result.

Round using IEEE 754 rounding rules.

Pack the result back into the 32-bit format.

## 3. Exponent Alignment Example

1.101 × 2^5
1.010 × 2^3

The exponent difference is 2, so the smaller mantissa is shifted right by two positions:

1.010 × 2^3 = 0.01010 × 2^5

Now both operands have the same exponent.

## 4. Normalisation and Rounding

A normalized result must have the form:

1.xxxxx × 2^n

If the result is 10.101 × 2^n, shift right once and increment the exponent:

1.0101 × 2^(n+1)

IEEE 754 normally uses round-to-nearest, ties-to-even. Guard, round and sticky (GRS) bits should be retained during intermediate calculations.

## 5. Debugging Faults

Fault 1 – Incorrect exponent alignment: The smaller exponent’s mantissa must be shifted by the exponent difference.

Fault 2 – Missing hidden bit: Normalized IEEE 754 numbers contain an implicit leading 1, so the mantissa is treated as 1.F.

Fault 3 – Incorrect normalization: A carry producing 10.xxxx requires a right shift and exponent increment.

Fault 4 – Early rounding: Rounding before normalization can produce incorrect results.

## 6. Optimisation

Parallel exponent comparison.

Barrel shifter for fast mantissa alignment.

Guard, round and sticky bits.

Leading-zero detection hardware.

Separate normalization and rounding stages.

Pipeline stages for high-throughput operations.

Accurate IEEE 754 operation = Exponent alignment + Mantissa calculation + Normalization + Rounding

# Q5. Debugging and Optimising an Instruction Execution Pipeline

## 1. Problem Understanding

A basic three-stage processor pipeline is:

IF → ID → EX

IF = Instruction Fetch, ID = Instruction Decode, and EX = Execute. Delays can occur because of data, control, and structural hazards.

## 2. Method Selected

Pipeline hazards can be resolved using:

Forwarding

Stalling

Instruction reordering

Branch prediction

Pipeline balancing

## 3. Data Hazard Example

I1: ADD R1, R2, R3
I2: SUB R4, R1, R5

Instruction I2 requires R1, which is produced by I1. This is a RAW (Read After Write) data hazard.

Forwarding can directly transfer the result from the EX stage of I1 to the EX stage of I2, avoiding unnecessary stalls.

## 4. Control Hazard

BEQ R1, R2, LABEL

The processor may not know the correct next instruction until the branch condition is evaluated. This creates a control hazard.

Possible solutions include branch prediction, delayed branching, early branch resolution, and pipeline flushing after a wrong prediction.

## 5. Instruction Reordering

Original:
I1: LOAD R1, [R2]
I2: ADD R3, R1, R4
I3: MUL R5, R6, R7

If I3 is independent of I1 and I2, it can sometimes be moved between them:

Optimised:
I1: LOAD R1, [R2]
I3: MUL R5, R6, R7
I2: ADD R3, R1, R4

This can reduce idle pipeline cycles provided the reordering does not change program behaviour.

## 6. Pipeline Performance Calculation

For k pipeline stages and n instructions, the ideal number of cycles is approximately:

Cycles = k + n - 1

For a 3-stage pipeline executing 10 instructions:

Cycles = 3 + 10 - 1 = 12 cycles

Without pipelining:

Cycles = 10 × 3 = 30 cycles

Ideal speedup:

Speedup = 30 / 12 = 2.5

Actual speedup is lower because of hazards, stalls and branch penalties.

## 7. Optimisation

Forwarding: Reduces data-hazard stalls.

Instruction Reordering: Places independent instructions between dependent instructions.

Branch Prediction: Reduces control-hazard penalties.

Pipeline Balancing: Prevents one stage from becoming the major timing bottleneck.

Efficient Hazard Detection: Automatically detects dependencies and inserts stalls only when necessary.

# Overall Conclusion

The five debugging problems demonstrate how arithmetic and processor performance can be improved through careful analysis of hardware algorithms.

| Problem | Main Fault | Optimisation |
| --- | --- | --- |
| Q1 – 2’s Complement | Incorrect overflow/sign handling | MSB-based overflow detection |
| Q2 – CLA | Carry logic delay | Parallel/hierarchical carry generation |
| Q3 – Booth | Incorrect shift/decision logic | Arithmetic shift and modified Booth |
| Q4 – IEEE 754 | Alignment/rounding errors | GRS bits + normalization |
| Q5 – Pipeline | Data/control hazards | Forwarding, reordering, prediction |

## Final Engineering Interpretation

The key principle across all five problems is to identify the critical operation causing incorrect results or delay, correct the underlying logic, and then optimise the critical path without unnecessarily increasing hardware complexity.

Debug → Verify → Optimise → Evaluate Performance