SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1 – ANALYTICAL PROBLEM SOLVING

CSA12 – COMPUTER ARCHITECTURE

| Student Name | P JASWANTH |
| --- | --- |
| Reg. No. | 19231243& |
| Department | ECE |

# Q1. Booth’s Multiplication: (−13) × 9

## 1. Problem Understanding

Booth’s algorithm performs signed multiplication using 2’s complement representation. The expected result is:

−13 × 9 = −117

Using 5-bit operands:

−13 = 10011₂
9 = 01001₂
A = 00000, Q = 01001, Q−1 = 0

## 2. Method Selection

Booth decision rules:

| Q₀Q−1 | Operation |
| --- | --- |
| 00 | No operation |
| 01 | A = A + M |
| 10 | A = A − M |
| 11 | No operation |

After every operation, perform an arithmetic right shift on [A,Q,Q−1].

## 3. Solution Process

| Iteration | Q₀Q−1 | Operation | A after operation | After arithmetic shift |
| --- | --- | --- | --- | --- |
| 1 | 10 | A=A−M | 01101 | A=00110, Q=10100, Q−1=1 |
| 2 | 01 | A=A+M | 11001 | A=11100, Q=11010, Q−1=0 |
| 3 | 00 | None | 11100 | A=11110, Q=01101, Q−1=0 |
| 4 | 10 | A=A−M | 00101 | A=00010, Q=10110, Q−1=1 |
| 5 | 01 | A=A+M | 10101 | A=11010, Q=01011, Q−1=0 |

A Q = 11010 01011 = 1101001011₂ = −117

## 4. Debugging and Optimisation

01 must mean A=A+M and 10 must mean A=A−M.

Use arithmetic, not logical, right shifts.

Initialize Q−1=0 and execute five iterations.

Modified Booth/Radix-4 can process two multiplier bits per cycle and reduce execution cycles.

Interpretation: The corrected result is −117, which matches the expected mathematical result.

# Q2. Restoring Division: 29 ÷ 5

## 1. Problem Understanding

Restoring division subtracts the divisor from the partial remainder and restores it when the subtraction becomes negative.

29 = 11101₂
5 = 00101₂
29 = 5×5 + 4

## 2. Method Selection

Use partial remainder A, divisor M and quotient Q. If A−M is negative, restore A and set the quotient bit to 0; otherwise retain the subtraction and set the quotient bit to 1.

## 3. Solution Process

| Iteration | A after shift | Operation | Final A | Q |
| --- | --- | --- | --- | --- |
| 1 | 000001 | Negative → restore | 000001 | 11010 |
| 2 | 000011 | Negative → restore | 000011 | 10100 |
| 3 | 000111 | A−M succeeds | 000010 | 01001 |
| 4 | 000100 | Negative → restore | 000100 | 10010 |
| 5 | 001001 | A−M succeeds | 000100 | 00101 |

Quotient = 00101₂ = 5
Remainder = 00100₂ = 4

## 4. Debugging

The faulty restore step occurs when a negative partial remainder is retained instead of restored. Correctly:

If A−M < 0: A=A+M and Q₀=0
If A−M ≥ 0: retain A−M and Q₀=1

## 5. Optimisation and Interpretation

Non-restoring division avoids immediate restoration and therefore reduces unnecessary operations. The corrected result is 29 ÷ 5 = 5 remainder 4.

# Q3. Non-Restoring Division

## 1. Problem Understanding

Non-restoring division allows the partial remainder to become negative. The next operation depends on the sign of the previous partial remainder. Since the question gives no numerical operands, 29 ÷ 5 is used for verification.

## 2. Method Selection

Previous A ≥ 0 → A=A−M
Previous A < 0 → A=A+M
New A ≥ 0 → Q₀=1
New A < 0 → Q₀=0
Final A < 0 → A=A+M

## 3. Solution Process

| Iteration | Sign before | Operation | A after | Q₀ |
| --- | --- | --- | --- | --- |
| 1 | + | A=A−M | −4 | 0 |
| 2 | − | A=A+M | −2 | 0 |
| 3 | − | A=A+M | +2 | 1 |
| 4 | + | A=A−M | −1 | 0 |
| 5 | − | A=A+M | +4 | 1 |

Quotient = 00101₂ = 5
Remainder = 00100₂ = 4

## 4. Debugging, Optimisation and Interpretation

The logic error occurs when the operation for an iteration is selected using the new remainder instead of the previous remainder. The final negative-remainder correction must also be included. Non-restoring division eliminates repeated restoration steps.

Verification: 5×5 + 4 = 29

# Q4. Arithmetic Right Shift in Booth Multiplication

## 1. Problem Understanding

Booth multiplication uses signed 2’s complement values. Therefore, the sign bit must be preserved during every right shift.

## 2. Debugging

−13 = 11110011₂

Logical right shift inserts 0 and corrupts the sign:

11110011 → 01111001

Arithmetic right shift copies the original sign bit:

11110011 → 11111001

## 3. Why Arithmetic Shift Is Required

Arithmetic shifting preserves sign extension. During Booth multiplication, the combined [A,Q,Q−1] register must therefore be shifted arithmetically after every operation.

## 4. Optimisation and Interpretation

A hardware arithmetic shifter can perform the sign-preserving shift efficiently. Logical shifting can convert negative intermediate values into positive values and produce an incorrect signed product; arithmetic shifting maintains correct 2’s complement representation.

# Q5. IEEE 754 Floating-Point Addition

## 1. Problem Understanding

IEEE 754 single precision has 1 sign bit, 8 exponent bits and 23 fraction bits. The exponent bias is 127.

Value = (−1)^S × 1.F × 2^(E−127)

The implementation error is adding mantissas before aligning their exponents.

## 2. Method Selection

Extract fields → Compare exponents → Align smaller exponent → Add/subtract mantissas → Normalize → Round → Pack

## 3. Numerical Solution: 5.5 + 2.25

5.5 = 1.011×2²
2.25 = 1.001×2¹

Align the smaller exponent:

1.001×2¹ = 0.1001×2²

1.0110
+ 0.1001
---------
  1.1111

1.1111×2² = 111.11₂ = 7.75

Therefore:

5.5 + 2.25 = 7.75

## 4. IEEE 754 Representation

5.5: 0 | 10000001 | 01100000000000000000000
2.25: 0 | 10000000 | 00100000000000000000000

## 5. Debugging and Optimisation

Align exponents before mantissa addition.

Use a barrel shifter for fast alignment.

Use guard, round and sticky bits for accurate rounding.

Use leading-zero detection for fast normalization.

Pipeline exponent comparison, alignment, addition, normalization and rounding for higher throughput.

Interpretation: Correct exponent alignment gives the accurate result 7.75.